Electrical Safety Audit Checklist

| Contractor: |  |  |  |  | Location: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Project Name: |  |  |  |  | Project Number: |  |
| Inspection Date: |  | Time: |  |  | College: |  |
| LACCD Regional Safety Manager: |  |  |  |  |  |  |

| DESCRIPTION |  | YES | NO | COMMENTS |
| --- | --- | --- | --- | --- |
| A. GENERAL GUIDELINES |  |  |  |  |
| 1. Only qualified persons trained to avoid electrical hazards of working on or near exposed electrical parts are permitted to work on or near such parts |  |  |  |  |
| 2. Qualified workers are trained to: |  |  |  |  |
| i. | Distinguish exposed live parts |  |  |  |
| ii. | Determine nominal voltage of live parts |  |  |  |
| iii. | Know and stay within clearance distances specified by OSHA for working with or near specific voltages |  |  |  |
| 3. Employees who are not qualified persons but work on, near, or with live exposed energized parts are trained in appropriate safety practices |  |  |  |  |
| 4. Live parts are de-energized before employees work on or near them |  |  |  |  |
| 5. Only qualified, trained persons using safety techniques and insulating equipment and tools work on equipment left energized due to design limitations or potential hazard increase from de-energizing |  |  |  |  |
| 6. Equipment that is de-energized but not locked or tagged out is treated as energized |  |  |  |  |
| 7. Control circuit devices such as push buttons are not used as sole means for de-energizing |  |  |  |  |
| 8. De-energized equipment with potential of employee exposure is locked and/or tagged out |  |  |  |  |
| 9. Re-energizing follows these steps: |  |  |  |  |
| i. | Qualified person verifies that tools and other devices are removed |  |  |  |
| ii. | Employees in area warned to stay clear |  |  |  |
| iii. | Locks and tags removed by employee who installed or under that person’s supervision |  |  |  |
| iv. | Qualified person removes locks or tags if installer is not available |  |  |  |
| 10. Lockout / tagout procedures are in written form and available to employees |  |  |  |  |
| 11. De-energizing follows these steps: |  |  |  |  |
| i. | Circuits and equipment disconnected from electric energy sources |  |  |  |
| ii. | Stored electric energy released |  |  |  |
| iii. | Stored non-electrical energy that could re-energize electric circuits is blocked or relieved |  |  |  |
| iv. | Locks and tags placed on each de-energizing disconnect device |  |  |  |
| v. | Locks alone used only by qualified persons and for de-energizing one circuit during one shift |  |  |  |
| vi. | Tags alone used only when locks cannot be and when equally safe as locks |  |  |  |
| vii. | Tags alone supplemented by at least one other safety measure |  |  |  |
| viii. | Locks not removable without undue force or tools |  |  |  |
| ix. | Tags state that unauthorized use and removal is prohibited |  |  |  |
| x. | Qualified person verifies de-energizing with test equipment |  |  |  |
| xi. | Test equipment checked before and after use when testing over 600 volts |  |  |  |
| B. ELECTRICAL POWER SYSTEMS |  |  |  |  |
| 1. Electrical control panels clearly identified and secured |  |  |  |  |
| 2. Access to panels limited to certified electrical specialists |  |  |  |  |
| 3. Grounding tested |  |  |  |  |
| 4. Explosion-proof and waterproof fixtures used in designated areas |  |  |  |  |
| 5. No exposed conductors at rear of switchboard |  |  |  |  |
| 6. Flexible extension cords not frayed or spliced |  |  |  |  |
| 7. Equipment operated and maintained by trained specialists |  |  |  |  |
| C. OVERHEAD LINES |  |  |  |  |
| 1. Overhead lines de-energized and grounded or protective measures in place before work starts near them |  |  |  |  |
| 2. Protective measures prevent employee body or conductive equipment from contacting lines |  |  |  |  |
| 3. Unqualified persons working near lines meet OSHA guidelines (10 feet for voltages to 50kV; 10 feet plus 4 inches for every additional 10kV over 50) |  |  |  |  |
| 4. Qualified persons working near energized lines use insulated protective equipment, or insulate energized part or stay within OSHA guidelines for distance from specific voltage ranges |  |  |  |  |
| 5. Vehicles or equipment with elevation capability maintain 10-foot clearance from energized overhead lines (plus 4 inches for every 10kV over 50kV) |  |  |  |  |
| 6. Exceptions limited to those permitted by standard: |  |  |  |  |
| i. | Vehicle in transit with lowered structure (4 feet plus 4 inches for every 10kV over 50kV) |  |  |  |
| ii. | Properly rated insulating barriers prevent contact with lines |  |  |  |
| iii. | Properly insulated aerial lift is used by qualified person |  |  |  |
| 7. Employees on ground do not contact vehicle or attachments without wearing proper protective equipment or being clear of uninsulated parts of vehicle |  |  |  |  |
| 8. Employees stay clear of ground equipment when line contact is possible |  |  |  |  |
| 9. Barricades or insulation protect employees from hazardous ground potentials where needed |  |  |  |  |
| D. ILLUMINATION AND CONFINED SPACES |  |  |  |  |
| 1. Adequate illumination in place before employees enter confined spaces with exposed energized parts |  |  |  |  |
| 2. Employees do not reach blindly into areas with possible energized parts |  |  |  |  |
| 3. Protective shields, barriers, or insulating materials protect employees working in confined spaces with exposed energized parts |  |  |  |  |
| E. CONDUCTIVE MATERIALS |  |  |  |  |
| 1. Work practices and insulating materials used to minimize hazard of employee body contact with conductive materials |  |  |  |  |
| 2. Portable ladders used around energized parts have non-conductive (non-metal) side rails |  |  |  |  |
| 3. Employees prohibited from wearing conductive jewelry or clothing unless insulated to become non-conductive |  |  |  |  |
| 4. Conductive cleaning materials prohibited around energized parts unless procedures in place to prevent electrical contact |  |  |  |  |
| F. PORTABLE ELECTRIC EQUIPMENT (Tools and Extension Cords) |  |  |  |  |
| 1. Extension cords connected to equipment are not used to raise or lower it |  |  |  |  |
| 2. Extension cords not fastened in ways that could damage insulation |  |  |  |  |
| 3. Portable equipment inspected before use for defects |  |  |  |  |
| 4. Defective or damaged equipment removed from service |  |  |  |  |
| 5. Attachment and plugs always match |  |  |  |  |
| 6. Attachment plugs and receptacles not connected or altered to prevent grounding |  |  |  |  |
| 7. Adapters that interrupt grounding not used |  |  |  |  |
| 8. Only electrical equipment and cords approved for use near water or conductive liquids are used in such locations |  |  |  |  |
| 9. Employees do not plug or unplug energized equipment with wet hands |  |  |  |  |
| 10. Wet or otherwise conductive equipment is handled only with insulating protective equipment |  |  |  |  |
| 11. Locking-type connectors properly secured after connection |  |  |  |  |
| G. PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |
| 1. Personal protective equipment appropriate to hazard provided: |  |  |  |  |
| i. | Non-conductive head protection |  |  |  |
| ii. | Eye and face protection |  |  |  |
| 2. Personal protective equipment used where required |  |  |  |  |
| 3. Safety signs and tags used to warn employees about electrical hazards |  |  |  |  |
| 4. Protective equipment inspected periodically and maintained in safe, reliable condition Last inspection date __ ___ |  |  |  |  |
| 5. Insulated tools and handling equipment used |  |  |  |  |
| 6. Ropes and hand lines are non-conductive |  |  |  |  |
| 7. Electrical panels clearly identified |  |  |  |  |
| 8. Protective shields, barriers, or insulating materials used as protection near exposed energized parts |  |  |  |  |
| 9. Barricades and/or attendants used where necessary to prevent or limit employee access to energized parts |  |  |  |  |
| 10. Electrical staff familiar with National Electrical Code (NEC) requirements |  |  |  |  |
| 11. All panels grounded |  |  |  |  |
| 12. Employees trained to carry long, conductive objects in a horizontal position |  |  |  |  |
| 13. Conduit, BX cable, etc., properly attached to supports and tightly connected to outlet and junction boxes |  |  |  |  |
| 14. Switches checked for evidence of overheating |  |  |  |  |
| 15. Electrical switches marked to show their purpose |  |  |  |  |
| 16. Motors clean and free of excessive dirt and oil |  |  |  |  |
| 17. Portable lights equipped with proper guards |  |  |  |  |
| 18. Explosion-proof fixtures provided in designated areas |  |  |  |  |
| 19. Electrical panel doors kept closed |  |  |  |  |
| 20. Drums of flammable solvents grounded in storage areas |  |  |  |  |
| 21. No employees who are not qualified perform designated electrical work |  |  |  |  |
| 22. Affected employees follow safety precautions when working around energized circuits and overhead lines |  |  |  |  |
| 23. Any unsafe acts by employees working on electrical processes or around electrical circuits? |  |  |  |  |

| INSPECTED BY (PLEASE SIGN) |  | PRINT NAME |  | DATE |
| --- | --- | --- | --- | --- |